TECHNICAL RIDER

1. CONTROL ROOM MONITORS

. The promoter will provide to 6 channels with 9 monitors and 6 graphics EQ like DN360 (high quality like Lacoustics or Nexo D&B...) on inserts sends.

. Provide one mixing desk with 32 channels and five sends. We need a good sound technician to mix monitors.

2. FOH AND PA SYSTEM

. Provide one mixing desk with 32 channels, 4 aux post (and 6 aux pre if there is not mixing desk for monitors.). trade : Midas, Yamaha (PM series) Soundcraft (Spirit forbiden).

Digital Mixing : contac us (Yamaha M7CL CL5 PM5D PM1D, Soundcraft VI, Midas Pro, Digidesign, Innovason or SD Digico)

Sound engineer : Fabien Aubert faaubert@yahoo.fr (00 33 (0) 6 10 13 42 52)

IF ANALOG MXING DESK :

. FX : 1 PCM91 Lexicon, 1 M3000, 1 DTWO TC

. 4 gates channels (2 DS 201)

. 11 channels of compression (2 DL241, 1 DPR 402, 1 DBX 160 and 1 DL 441)

. 1 graphic EQ Klark technics or XTA or BSS to adjust the main system

The power of the PA system shall be chosen according to the venue and installed at the best higher. Line array in priority with skilled technician (Lacoustic dVDOSC, KARA, KUDO, KIVA, Nexo S12 GEO D, Adamson or EAW)

3. MICROPHONES

See at the end of document : PATCH LIST (this bellow)

4. BACKLINE

. Provide risers for drums (2x3 m) and a table for DJ (L:1,5 / l: 0,7 / h :0,75 )

. Something to get up the amplifiers

5. ELECTRIC POWER FOR BACKLINE

. 6 power supplies 16A with isolated phase and earth

6. PLAN OF STAGE

. See the document joins.

7. CONTACT Sound  Fabien AUBERT : 00 33 (0) 6 10 13 42 52 //  faaubert@yahoo.fr

Update : 2016/05

PATCH LIST

|  | Channels | Micros | # Stands | Inserts | Musicians |
| --- | --- | --- | --- | --- | --- |
| 1 | BD | D6 / B 52 / B91 / M88 | Sm | DL 241 | Nicolas |
| 2 | SN top | B 57 | Sm | DL 241 |  |
| 3 | SN bottom | SM 57 | Sm | DS 201 |  |
| 4 | HH | SM81 / C451 / KSM137 | Sm |  |  |
| 5 | Tom high | E904 / Beta98 |  | DS 201 |  |
| 6 | Tom mid | E904 / Beta98 |  | DS 201 |  |
| 7 | Floor tom | E904 / Beta98 |  | DS 201 |  |
| 8 | OH | KM184 / SM81 / C451 | Lg |  |  |
| 9 | OH | KM184 / SM81 / C451 | Lg |  |  |
| 10 | Bass DI | DI BSS / KT |  | Dbx 160 | Vincent |
| 11 | Bass mic | B 52 / RE20 / MD421 | Lg |  |  |
| 12 | Guitar | BF509 / SM57 | Sm | DPR 402 | Simon |
| 13 | DJ L | DI BSS / RADIAL / KT |  | DL 241 | Fuf |
| 14 | DJ R | DI BSS / RADIAL / KT |  | DL 241 |  |
| 15 | Saxophone | B 98 HF, we provide |  | DL 441 with group | Bino |
| 16 | Trumpet / Trombone | B 98, we provide |  |  | Laden |
| 17 | Trumpet | B 98, we provide |  |  | Tim |
| 18 | Vocal Simon | SM 58 | Lg | DL 441 with group |  |
| 19 | Vocal Fuf | SM 58 | Lg |  |  |
| 20 | Vocal Vincent | SM 58 | Lg |  |  |
| 21 | Vocal Bino | SM 58 | Lg |  |  |
| 22 | Lead Vocal | SM 58 | Lg | DPR 402 | Julien |
| 23 | Guest | SM 58 | Lg |  |  |
| 24 | Return PCM 91 L |  |  |  |  |
| 25 | Return PCM 91 R |  |  |  |  |
| 26 | Return M3000 L |  |  |  |  |
| 27 | Return M3000 R |  |  |  |  |
| 28 | Return Dtwo L |  |  |  |  |
| 29 | Return Dtwo R |  |  |  |  |
| 30 | Talk back |  |  |  |  |

Auxiliary (if monitors from FOH) :

SOUND

Fabien Aubert     0033 (0)6 10 13 42 52   faaubert@yahoo.fr

ACCOMODATIONS

Vincent Combet  0033 (0)6 62 27 62 78   vincentcombet@gmail.com

Benoît Templier   0033 (0)6 82 91 56 10  bino.joke@gmail.com

Modifications are of course possible according to your

possibilities.

Please send us the technical rider of your place and contact us

in order to be appropriate of the changes.

Thanks and have a good day !

1   – Monitor Guitar

2   – Monitor Lead

3   – Monitor Brass

4   – Monitor DJ

5  – Monitor Drums

6  – Monitor Bass

7   – PCM 91

8   – M3000 A

9   – M3000 B

10 – DTWO

Update : 2016/05